| Contractor Risk Management Planning Reference Tool – Construction / Maintenance |  |  |  |
| --- | --- | --- | --- |
| 1. Is it a construction project (over $250,000)? | - WHS Management Plan is required - Does WHS Accreditation Scheme apply? (over $4 million funded by the Australian Government) |  | UNE contract engager / supervisor to complete Contractor Safety Evaluation Checklist |
| 2. Does the job involve construction work ($ does not matter)? | - White Cards (General Construction Induction) - SWMS for all high risk activities |  |  |
| 3. Does the work involve high-risk work activities? Note: UNE requires a SWMS to be developed for high-risk activities for both construction and maintenance work. | - Risk of fall > 2m - Demolition of an element of a structure that is load bearing - Work on a telecommunication tower - Disturbance of Asbestos - Involves structural alterations or repairs that require a temporary support to prevent collapse - Confined space - In or near a shaft, tunnel or trench with a depth > 1.5m - Involves the use of explosives - On or near pressurised gas mains or piping - On or near chemical, fuel or refrigerant lines - On or near any energised electrical installation or service - An area that has a contaminated or flammable atmosphere - Tilt-up or precast concrete elements - On, in or adjacent to a road or other traffic corridor that is in use, other than pedestrians - Movement by powered mobile plant - An area that has artificial extremes of temperature - In or near water or other liquid where there is a risk of drowning - Diving work |  | UNE contract engager / supervisor to review: - Check all high risk work is identified - Is it site specific - Describes step by step work method (hazards & risks) - Includes specific control measures - Details who is responsible for monitoring SWMS - Is developed in consultation with appropriate personnel - All workers have signed SWMS - Process for communicating changes to SWMS If you require assistance / advice, please contact EBE or the WHS Team. |
| 4. Does the work require licences? Refer to page 3 for a complete list of licences | - Asbestos licence (class A – friable) (class B- non-friable) - High risk work licences - Contractor licence - Builder - Electrical licences - Demolition licence - Fire Protection licences |  | UNE contract supervisor is required to confirm the licence matches the scope of works. If you require assistance / advice, please contact EBE or the WHS Team. |
| 5. Will the work require permits to work to be completed? | - Asbestos Removal - Working at Heights - Roof Access | - High Voltage Access - Electrical Work - Crane Site | UNE permit approvals are issued by the EBE Directorate |
|  | - Critical System Impairment - Fire System Isolation / Hot Work - Excavation >300mm / Penetration >25mm |  |  |

| 6. Are there any specific legislative obligations to be met (e.g. CoP, AS) | - Include in specification / purchase order - Accessibility hazards | If you require assistance / advice, please contact the WHS Team. |
| --- | --- | --- |
| 7. Psychosocial hazards | - Remote or isolated work - Fatigue - Unclear work demands - Inadequate key information provided to contractor - Exposure to extreme risks/hazardous environments - Violence and aggressive staff/student behaviour - Bullying - Discrimination or harassment - Conflict or poor working relationships/interactions - Environmental stressors *Refer to UNE Contractor Psychosocial Risks Guide for additional information on risks and controls | - Detailed work specifications in contracts - Psychosocial hazards and controls included in contractor inductions - Incident reporting and management system - Risk controls documented in SWMS or documented in UNE protocol/s. |
| 8. Clearly determine responsibilities of contractor / UNE | - Shared duties are clearly defined in specification / purchase / work order | Contact EBE or WHS team. |
| 9. Induction | - Contractors have access to induction prior to arrival at site | Online Contractor Induction |
| 10. Communication / consultation requirements | - Site / area specific induction - Commencement / Kickoff meeting - Are scheduled meetings required to monitor progress? - Consider frequency of contractor observation audits - Are there any critical control points (e.g. risk unable to be assessed initially – hold points) - Communication between the UNE Contract Supervisor and onsite Contractor available at all times (e.g. mobile phone numbers) - Is the contractor aware that they must consult the UNE Contract Supervisor prior to changing SWMS / scope of works | UNE contract supervisor to coordinate |
| 11. Works completion | - Inspection of area to confirm it is free of hazards - Documentation / training of new installations have been organized / completed. | UNE contract supervisor to conduct |

| UNE risks and hazards What UNE hazards and risks may impact on the contractor? These need to be communicated to the contractor: - Access - Electrical - Chemicals/ Asbestos - Pressurised equipment - Radiation - Restricted area (anatomy,laboratories, etc.) - UNE supplied plant / equipment – highlight risks and provide SOP / SWP’s risk assessments, rules, manufacturer’s instructions - Risk of fall (UNE specific controls / hazards) – details of anchor points, roof access rules, etc. - Student / staff activity affecting work - Water risks - Extreme temperatures - Traffic / working on UNE roads / carparks - Hot work options - Waste management - Emergency management - Working in isolation - Impacts on ventilation - Notification requirements (to occupants affected by works) | Safety documentation that may be required depending on the work: - Construction WHS Management Plan - Project Management Plan - SDS chemicals, equipment manuals - Safe work method statements for high risk construction work – specific to work - Job specific risk assessments - A Construction/Demolition Plan - Permits to work - Copies of licences/certifications - Insurance documents - Specialist design or technical data - Safety in design reviews - Monitoring certificates (e.g. asbestos / mould) |
| --- | --- |
| # High risk work licences (WHSL): BA – Boiler Advanced BS – Boiler standard ES – Reciprocating steam engine TO – Turbine operation C0 – Slewing mobile crane over 100t C1 – Slewing mobile crane up to 100t C2 – Slewing mobile crane up to 20t C6 – Slewing mobile crane up to 60t CB – Bridge and gantry crane CD – Derrick crane CN – Non-slew mobile crane CP – Portal boom crane CS – Self-erecting tower crane CT – Tower crane CV – Vehicle loading crane DG – Dogging HM – Material hoist HP – Personnel and material hoist WP – Boom elevated work platform LF – Forklift truck LO – Order-picking forklift truck PB – Concrete placing boom Vehicle RB – Basic rigging RI – Intermediate rigging RA – Advanced rigging RS – Reach stacker SB – basic scaffolding SI – Intermediate scaffolding SA – Advanced scaffolding | Electrical Licences Electrical mechanic licence Electrical linesperson licence Electrical fitter licence Electrical joiner licence Restricted electrical work licence Electrical work training permit # Asbestos Asbestos – class A (friable) Asbestos – class B (non-friable) Demolition Licence |